# Guidance

# On the Breakdown of the

# Fire Alarm System

# Document 12

Guidance on the Breakdown of the Fire Alarm System

- Introduction

The Premises staff will be aware if the fire alarm is not working during their normal daily school checks. If the fire alarm panel/system is not working this means that the school occupants are now at a higher risk and potential concerns that an undetected fire may prevent the occupants from evacuating safely.

The following areas must be incorporated within the Headteachers/Premises risk assessment so that a safe working environment can be maintained whilst the fire alarm system is inoperative for a short time (e.g. no more than 2 or 3 working days maximum). In all circumstances when the alarm will be inoperative for longer than 2-3 days, the school should ensure that their fire risk assessment is reviewed by a competent person.

It should be noted that the following guidance might require enhancement in terms of additional measures that need to be considered and introduced appropriate to the school or premises involved.

- Risk Assessment

The school should complete a risk assessment for this risk. An example risk assessment is included in Appendix 1 of this guidance. The urgent requirement is for the school’s fire alarm company to complete an urgent site visit to investigate what the alarm failure cause is and to fix it ASAP. It is the time when the breakdown of the fire alarm panel/system to when it is fully fixed and fully operational, that needs to be fully risk assessed with appropriate control measures.

- Alternative Means Of Giving Warning In The Event Of A Fire

Handbells, air horns, klaxons or whistles can be deployed as a temporary means of giving warning in the event of a fire. However, the assessment of what is needed to be able to alert the school occupants must be appropriate to the size of the building, the number of floors etc. Also, a sufficient number of handbells, air horns, klaxons or whistles would have to be readily available to ensure that adequate provision is made on each floor area.

In most circumstances, at least two handbells or air klaxons would have to be available on each floor (at either end of the floor adjacent to fire exit staircases for example). However, a travel distance of 30 meters for each handbell or klaxon should not be exceeded. It is suggested that the handbell or klaxon be located in classrooms and areas out of the way of the children but still accessible by staff and preferably sign-posted to eliminate any delay. The handbells or klaxons can be supported by whistles. It is recommended that a sufficient number of whistles should be purchased for issue to staff based on all floors of the building.

(Continued)

- Appointment Of Additional Floor Fire Wardens

During this period the fire risk assessment control measure would be to recommend that during the temporary failure of the permanent alarm system, additional floor Fire Wardens must also be appointed. Fire Wardens in this instance would be deployed to patrol floors, rooms and other areas

To ensure that:

- A fire, or a smell of smoke or burning, can be investigated and if safe to do so,          dealt with as rapidly as possible.

- That the backup alarm can be raised as rapidly as possible with the use of

Klaxons/Air horns.

- That, if safe to do so, a fire can be extinguished.

- So that basic fire prevention duties are carried out. (E.g. arranging for           obstructions and rubbish to be removed immediately, ensuring that fire doors          are closed, routine hazard spotting etc.)

- So that they can assist disabled persons from the premises

To maintain and fulfil the above duties, ‘Fire Wardens’ will have to be excused from their normal contracted duties. The Headteacher/Premises Manager must also ensure that a sufficient number of “Fire Wardens are appointed to ensure that:

At least one warden is on patrol on each floor of the building (depending on floor area).

That sufficient cover is available so that wardens can be relieved for normal and necessary break periods. (NB: consider cover for toilet breaks, smoking breaks, mid-morning refreshments, and lunch breaks etc.)

That all wardens on patrol have access to a phone so that they can alert the Fire Brigade in the event of a fire, or suspicion of a fire.

- Supplementing The Temporary Means of Giving Warning In The Event of a Fire

Additionally, the assessment must ensure that the revised procedure for raising the alarm (whilst the permanent alarm system is inoperable) is adequate and suitable for the premises.

All teaching staff should be issued with a whistle during the period that the permanent fire alarm is inoperable, to augment the temporary means of giving warning in the event of a fire that has been put into place by the school.

For example:

In the event of a fire (or on the suspicion that there is a fire) being discovered by an appointed floor fire warden, he/she should immediately raise the alarm via the nearest handbell or klaxon (as applicable).

All teaching staff whilst evacuating their individual classrooms or teaching areas would supplement the Fire Warden’s alarm by blowing their whistle whilst evacuating the school building. All other school staff, pupils and visitors etc. on hearing a handbell and/or whistle being sounded shall immediately commence to evacuate the building.

Vice-versa, In the event of a floor fire warden hearing a whistling sound, he/she shall immediately supplement the alert by sounding the nearest available handbell or klaxon (as applicable). The emphasis must be on ensuring that an adequate and effective means of giving a clear, sufficiently loud and distinct form of warning in the event of a fire is in place. The type and form of warning chosen must be distinct and unambiguous!

The temporary arrangements that are put into place must be notified to staff, pupils, contractors and members of the public (as applicable) resorting to the building, to eliminate any potential confusion regarding the temporary means of giving warning in the event of fire.

- Communication Between Floor Fire Wardens And The Person(S) Responsible For Calling The Fire Brigade

The Headteacher/Premises Manager must also ensure that suitable and adequate arrangements are evaluated and put into place to ensure that there is effective and speedy communication between floor fire wardens and school support/reception staff (or another responsible school/premises staff – whose duties would include calling the Fire Brigade in the event of a fire). In this case, the use of mobile phones would be permitted.

The chosen method of communication must be appropriate, clear and unambiguous and staff must be informed of the temporary arrangements in writing!

- Communicating The Revised Evacuation Procedure

During the period that the permanent fire alarm system is inoperable, the school’s/premises fire evacuation procedure would have to be revised to take into account and notify the temporary method of warning that will be given in the event (or suspicion of) a fire.

The revised fire instructions must be:-

- Issued to all staff

- Displayed (to replace existing fire instruction/fire action notices) throughout the school, in prominent positions, particularly adjacent to handbells and/or klaxons

- Communicated to all pupils (during school assembly for example) and all school visitors

- Disabled staff and pupils: The breakdown of the fire alarm system could pose additional risks in the evacuation of temporary/permanently disabled staff. A review of their personal emergency evacuation plan (PEEP) detailing any additional arrangements put in place during the fire alarm breakdown to assist disabled colleagues to evacuate the premise

Appendix 1

Risk Assessment

| School Name |  |
| --- | --- |

Task Or Activity Covered By This Assessment

- Breakdown or failure of the school fire alarm system.

Hazards Identified (Potential to cause harm)

- The outbreak of a fire will remain undetected due to the failure of automatic fire detection equipment. - Staff/visitors unable to actuate fire alarm system to give warning in the event of a fire.

Consequences of the Hazard (i.e. What is the potential harm)

- Potential for staff, pupils and visitors sustaining fatal or serious injury as a result of an undetected outbreak of fire. - Potential for fire development due to the inability to activate the fire alarm system and as a result staff, pupils and visitors being placed at risk (fatal or serious injury).

Persons At Risk

- All staff, pupils and visitors

Current Controls (Including legal compliance)

- School to enact the written procedure for this event. - Additional floor fire wardens (either staff or temporary premises staff) are to be engaged to undertake fire-watching duties, including floor patrols. The fire wardens appointed must not be engaged in any other duties apart from fire-watching. - Fire wardens are to raise warnings with whistles in the event of a fire being discovered whilst patrolling. - Teaching staff to be issued with whistles, so that they can augment the fire warden’s air horns in the event of a fire. - In the event of a warning (bells and whistles) staff, pupils and visitors will evacuate the building using the designated escape routes and staircases - On the evacuation of the building report to the designated fire assembly point at the rear of the main playground. - Fire wardens Administration will notify the Fire Brigade of the fire or assign the nearest available person with a working mobile to do this. - The school will restrict any high-risk works, such as contractors working with gas/flame/hot works

| Severity of the Outcome (Tick or outline as appropriate) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Minor/slight injury or illness | Injury/illness incurring time loss from work/subjects |  |  | Serious injury or illness |  | Life-threatening or fatal injury or illness |
| Probability Of Harm (Tick as appropriate) |  |  |  |  |  |  |
| Unlikely/ Improbable |  | Likely | Highly Probable |  | Certainty |  |

| Final Assessment Of Risk (Where appropriate) |  |  |  |
| --- | --- | --- | --- |
| Low | Medium | High | Very High |

Action Required (Continue on a separate sheet if required)

- The current controls that have been put into place by the school are to be strictly adhered to. - A copy of this assessment shall be emailed to all staff. - Additional controls are to be considered on the introduction of any new school, or the introduction of new equipment or electrical/mechanical apparatus etc. - This assessment must be immediately reviewed in the event of an incident. - Confirmation of repair to fire alarm from school fire alarm company Action to be taken by _____________________________ To be completed by

Arrangements For Occupational Health Monitoring (Where appropriate)

NOT APPLICABLE

| Guidance Version Control |  |  |
| --- | --- | --- |
| Name Of Document | Version | Last Review date |
| Guidance on the Breakdown of the Fire Alarm System | NASPM V3 | August 2024 |

Copyright © 2024 NASPM

All rights reserved.

No part of any NASPM document may be reproduced, distributed, or transmitted in any form or by any means, including photocopying, recording, or other electronic or mechanical methods, to any individual, school, or organisation outside of the school named in the school membership agreement.

Should membership terminate, no part of any NASPM document may be used, reproduced, distributed or transmitted by any individual, school, or organisation.